## TRANSMISSION LINES & WAVEGUIDES – ECA1501

EXPERIMENT – 3

# Designing a Quarter wave transformer by varying following Parameters a) frequency b) input impedance c) characteristic impedance

| Name | Noorbasha Chotu | Reg. No. | 192412383 |
| --- | --- | --- | --- |
| Date | 14/10/2027 | Experiment No. | 3 |

### AIM

To design a quarter-wave transformer for an input impedance (Zin) of 50 ohms and a load impedance (ZL) of 100 ohms and to analyze the transformer's performance as the operating frequency is varied across {100 MHz, 200 MHz, 300 MHz, 500 MHz }.ZL =Zo.

### SOFTWAREREQUIRED:

SCILABVersion6.1.1

### PROCEDURE:

- Open SCILAB software

- Open file-new

- Put the input values

- Type the program

- Execute the program

- End the program

### PROGRAM:

clear;clc; Zr=100;s=9;d=0.1; Zs=50;

Ro=sqrt(Zs*Zr);f=500*(10^6);r=d/2; Zof=276*log10(s/r);

Zoq=sqrt(Zr*Zof); L=300/(f*(10^-6));

printf("length of lambda/4 transformer = %f meters\n",L);

printf("characteristic impedance of quarter wave line = %f ",fix(Ro*10000)/10000);

### TEST CASE 1

Design a quarter-wave transformer for an input impedance (Zin) of 50 ohms and a load impedance (ZL) of 100 ohms. Analyze the transformer's performance as the operating frequency is varied across {100 MHz, 200 MHz, 300 MHz, 500 MHz}.

OUTPUT TABLE 1

ZR=100 ohms Zin =50 ohms

| ZS ohms | ZR ohms | Ro ohms |
| --- | --- | --- |
| 50 | 100 | 70.71 |

ZR=100 ohms Zin =50 ohms

| Frequency (MHz) | Length of λ/4 transformer (meters) |
| --- | --- |
| 100 | 0.7500 |
| 200 | 0.3750 |
| 300 | 0.2500 |
| 500 | 0.1500 |

### RESULT:

A quarter-wave transformer for an input impedance (Zin) of 50 ohms and a load impedance (ZL) of 100 ohms is designed and the transformer's performance analyzed for the varying operating frequency is varied across {100 MHz, 200 MHz, 300 MHz, 500 MHz }.

\

### TEST CASE 2

To design a quarter-wave transformer a ftixed operating frequency of 2 GHz and a load impedance (ZL) of 100 ohms and evaluate its performance as the input impedance (Zin) varied .To validate that the transformer's characteristic impedance (Ztransformer= Ro= sqrt(Zin⋅ZL) changes with Zin.

OUTPUT:

ZR=100 ohms Variable :Zin

| Zin | Ro |
| --- | --- |
| 25 | 50.00 |
| 50 | 70.71 |
| 75 | 86.60 |
| 100 | 100.00 |

Z_L=100,Omega), (f=2,GHz)

| Characteristic Impedance \(R_o\) (Ω) | Input Impedance \(Z_{in}\) (Ω) |
| --- | --- |
| 50 | 25 |
| 70.71 | 50 |
| 86.60 | 75 |
| 100 | 100 |

### RESULT:

A quarter-wave transformer is designed at a fixed operating frequency of 2 GHz and load impedance (ZL) of 100 ohms and its performance is evaluated for various input impedance values. The variation of transformer’s characteristic impedance with Zin is also validated..