# TRANSMISSION LINES & WAVEGUIDES – ECA1501

## EXPERIMENT – 4

| Name | Noorbasha Chotu | Reg. No. | 192412383 |
| --- | --- | --- | --- |
| Date | 16/10/2026 | Experiment No. | 4 |

Strategize Impedance Matching With Characteristic Impedance And Load Impedance Through The Measurement Of A) Location Of The Stub B) Length Of The Stub

### AIM

To design strategize impedance matching with characteristic impedance and load impedance through the measurement of a) location of the stub b) length of the stub.

### SOFTWAREREQUIRED:

### SCILABVersion6.1.1

### PROCEDURE:

### ●Open SCILAB software

### ● Open file-new

### ● Put the input values

### ● Type the program

### ● Execute the program

### ● End the program

PROGRAM:

clc;

clear;

Zo = 60;

f = 100 * 10^6;

lambda = 3 * 10^8 / f;

printf("ZR(Ohms)\tPoint of attachment(cm)\tStub length(cm)\n");

for ZR = 100:50:350

theta = 30 * %pi / 180;

ZL = ZR * (cos(theta) + %i * sin(theta));

zL = ZL / Zo;

yL = 1 / zL;

g = real(yL);

b = imag(yL);

A = g - g^2 - b^2;

B = -2 * b;

C = g - 1;

D = B^2 - 4 * A * C;

if D >= 0 then

t1 = (-B + sqrt(D)) / (2 * A);

d = atan(t1);

if d < 0 then

d = d + %pi;

end

y = (yL + %i * tan(d)) / ...

(1 + %i * yL * tan(d));

bs = -imag(y);

ls = atan(bs);

if ls < 0 then

ls = ls + %pi;

end

printf("%d\t\t%.2f\t\t\t%.2f\n", ...

ZR, lambda*d/(2*%pi)*100, ...

lambda*ls/(2*%pi)*100);

else

printf("%d\t\tNo real solution\n", ZR);

end

end

### TEST CASE 1

### Optimize impedance matching using single stub matching by finding the location and length of the stub for the values of load impedance of 100 30o to 350 30o keeping the characteristic impedance Zo= 60 Ω constant .

OUTPUT TABLE 1

Variables ZR;

| Input | Output |
| --- | --- |

Constants Zo = 60Ω; f=100*(10^6);

| ZR Ohms. angle 30o | Point of attachment | Length of short-circuited stub (cm) |
| --- | --- | --- |
| 100∠30° | 28.46 | 141.82 |
| 150∠30° | 39.45 | 124.77 |
| 200∠30° | 45.12 | 115.55 |
| 250∠30° | 48.76 | 109.70 |
| 300∠30° | 51.35 | 105.61 |

RESULT:

Impedance matching using single stub matching is optimized by calculating the location and length of the stub

or varying values of load impedance keeping the characteristic impedance Zo= 60 Ω constant .

### TEST CASE 2

Optimize impedance matching using single stub matching by finding the location and length of the

stub for the values of characteristic impedance ranging from 600Ω to 1800 Ω. Keeping the load

impedance ZR= 100 30o constant .

OUTPUT:

| Input | Output |
| --- | --- |

Variables Z0;

Constants ZR= 100 30o Ω; f=100*(10^6);

| Z0 Ohms. angle 30o | Point of attachment | Length of short-circuited stub (cm) |
| --- | --- | --- |
| 600 | 128.67 | 19.87 |
| 900 | 132.98 | 15.76 |
| 1200 | 135.48 | 13.44 |
| 1500 | 137.15 | 11.91 |
| 1800 | 138.37 | 10.81 |

RESULT:

Impedance matching using single stub matching is optimized by calculating the location and

length of the stub for varying values of characteristic impedance keeping the load impedance

constant .